Baptist Lui Ming Choi Secondary School

# F. 5 Physics

# Circuit Worksheet 1

Finish the following tasks with the aid of Electronics Workbench Demo.

Consult the handout Electronics Workbench Demo reference when required.

## Task 1	Resistance of a resistor

Change the internal resistance of the ammeter and voltmeter to be 1 Ω and 1 kΩ respectively.

Change the resistance of the resistor R to be 500Ω.

The battery is of 12 V. With the aid of Electronics Workbench Demo connect up the following circuits and

calculate the resistance of the resistor R (= 500 Ω) using the readings measured by the meters..

Circuit A									Circuit B

V = __________ V						 V = __________ V

I = ___________ mA						 I = ___________ mA

R = VI= _____________Ω				 	R = VI= _____________Ω

Which circuit give a better value of R (closer to the real value of R) ?		Circuit  _______

### Reasoning

Rv – internal resistance of the voltmeter			Rv – internal resistance of the ammeter

Ir – current passing through the resistor 			Vr – voltage across the resistor

Iv – current passing through the voltmeter			Va – voltage across the ammeter

For  circuit  A	 :						For  circuit  B :

(1)  Ammeter measures the current through R and	(1)  Voltmeter measures the voltage across R and voltmeter								ammeter

(2)  Voltmeter measures the voltage across R 		(2)  Ammeter measures the current through R

(3)  When R  Rv (in order),   Ir  Ir (same V),	(3)	When R >> Rv,   Vr >> Va (same I),

so the measured I (=Ir + Ir) > Ir, and				so measured V (=Vr + Va)  Vr, and

calculated R (= V / I ) is smaller than the real R.	calculated R (= V / I ) is close to real R

.

Now the calculated R is of the order of hundred Ohm, close to the internal resistance of the voltmeter and much greater than the internal resistance of the ammeter, so circuit ________ should be used.

When R is close to the internal resistance of ammeter and much smaller than internal resistance of voltmeter, circuit __________ should be used. (Why?

## Task 2	Change in ammeter and voltmeter reading in different circuits

Change the internal resistance of ammeter and voltmeter to be 1 MΩ.

Put down increases, decreases or unchanged for statements 1 – 8 when the switch S1 in circuit

A and B are opened.

Circuit A								  Circuit B

Circuit A		Circuit B

- The reading of ammeter A1.					  ____________	____________

- The reading of ammeter A2.					  ____________  ____________

- The reading of ammeter A3.					  ____________   ___________

- The reading of the voltmeter V1. 				  ____________	____________

- The brightness of bulb P.						  ____________	____________

- The brightness of bulb Q.					  ____________	___________

- The brightness of bulb R.					      ------		____________

- The power output from the battery.		 		  ____________	____________

- When switch S2 in circuit A and B are opened, what should be the readings in all the

meters? 		_____________

Explain the above in terms of the connection of the circuits (simple parallel, a mix of series and parallel).

#### END  OF  WORKSHEET